# DSP Motor Signal Maths

# Definitions

Use i for the analysis interval and n for the discrete sample index inside that interval.

N_i = number of samples in interval i, f_s = sample rate, W = moving-average window length, and τ = order-band tolerance fraction.

# Acceleration Magnitude Mean

a_mag^(i)[n] = √(a_x^(i)[n]^2 + a_y^(i)[n]^2 + a_z^(i)[n]^2)

μ_a_mag^(i) = (1 / N_i) Σ_(n=0)^(N_i-1) a_mag^(i)[n]

# Gyro Magnitude Mean

g_mag^(i)[n] = √(g_x^(i)[n]^2 + g_y^(i)[n]^2 + g_z^(i)[n]^2)

μ_g_mag^(i) = (1 / N_i) Σ_(n=0)^(N_i-1) g_mag^(i)[n]

# Temperature Moving Average

T_MA^(i)[n] = (1 / W) Σ_(k=0)^(W-1) T^(i)[n-k]

# Temperature Moving Average Gradient

∇T_MA^(i)[n] ≈ (T_MA^(i)[n] - T_MA^(i)[n-1]) · f_s

# Fundamental Frequency and Magnitude

For one discrete signal x^(i)[n], which may be one acceleration axis or a combined acceleration spectrum input:

X^(i)[m] = | Σ_(n=0)^(N_i-1) x^(i)[n] · e^(-j 2πmn / N_i) |

f^(i)[m] = m · f_s / N_i

m_fund^(i) = arg max_(m ∈ M_f) X^(i)[m]

f_fund^(i) = f^(i)[m_fund^(i)]

A_fund^(i) = X^(i)[m_fund^(i)]

# BPFO and BPFI Order Bands

Using the current order-band settings in the repo:

BPFO_low^(i) = (3 · f_fund^(i)) · (1 - τ)

BPFO_high^(i) = (5 · f_fund^(i)) · (1 + τ)

BPFI_low^(i) = (5 · f_fund^(i)) · (1 - τ)

BPFI_high^(i) = (7 · f_fund^(i)) · (1 + τ)

# BPFO and BPFI Band RMS

R_BPFO^(i) = √((1 / K_BPFO) Σ_(m ∈ B_BPFO^(i)) X^(i)[m]^2)

R_BPFI^(i) = √((1 / K_BPFI) Σ_(m ∈ B_BPFI^(i)) X^(i)[m]^2)