# AI / Data Processing Strategy

## Signals

Table 1 shows the data structure that will be received via the wired serial comm link from the MCU. Using Python, each column of sensor data will be read into 2-second buffer arrays, of which the most recent second will be used to build the time and frequency signals seen in Tables 2 and 3. See Appendix II for each signal’s derivation.

| Sample | acc_X | acc_Y | acc_Z | gyr_X | gyr_Y | gyr_Z | ambi | loc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.03 | 0.04 | 0.06 | 0.11 | 0.33 | 0.66 | 25 | 29 |
| 2 | … | … | … | … | … | … | … | … |

Table 1 – Sensor Data Structure

| Signal | Description |
| --- | --- |
| accMag | The magnitude of the x, y and z accelerometer axes. Combines 3 columns of data into a single signal. RMS will be used for signal conditioning [2]. |
| gyroMag | The magnitude of the x, y and z gyroscope axis. Combines 3 columns of data into a single signal. Again, RMS will be used for signal conditioning [2]. |
| ambi | The ambient temperature read from the MPU 6050 sensor. A moving average is used to capture trend. |
| ambiGrad | The gradient of the ambient temperature moving average. Used to determine the rate of change in ambient temperature at any given time. |
| local | The moving average of the DS18B20 temperature sensor at a critical location. |
| localGrad | The gradient of the DS18B20 sensor moving average to determine the rate of change in the targeted area at any given time |

Table 2 - Time Domain Signals

| Signal | Description |
| --- | --- |
| domMag | The magnitude of the dominant frequency over time. Increasing or decreasing magnitude can be indicative of different motor loads. |
| domFreq | The frequency of the dominant frequency over time. Similarly, this can also be indicative of changes to the motor loading. |
| Energy1_A (0-499hz) | Energy of low frequency band, typically indicative of rotor health and alignment [ref] |
| Energy2_A (500-1499hz) | Energy of the mid frequency band, typically indicative of ____ [ref] |
| Energy3_A (1500-3000hz) | Energy of the high frequency band, typically indicative of bearing health. [ref] |

Table 3 - Frequency Domain Signals

These tables outline a set of simple and descriptive signals. It is possible to derive more complex and deeper signals, however, for prototyping and proof of concept, the set of signals is deliberately kept simple and small to reduce data blowup when using multiple sensors.

## Machine Learning Strategy.

A supervised machine learning (ML) model will be implemented using PyTorch to generate a set of probabilities during motor sensing. The probabilities will define a likelihood that a failure state is occurring at any given moment of sensed data

Table 4 outlines the artificially induced failure states that the model will be trained on

| Failure | Description |
| --- | --- |
| Loose Mounting | Mounting screws/bolts and brackets will be systematically loosened. For example, if 4 bolts hold the motor in place, sensor data will be capture when 1, 2, 3, and 4 bolts are loosened. Targeting vibration sensing |
| Max and min loading | Recording the motor running at max or above max rated load to simulate strain in over loading conditions. Conversely, running the motor with little or no load. Targeting overheating and a shifting domMag and domFreq. |
| Off Axis | Tilting the motor off its mounting axis in the Y and Z directions. Targeting gyroscopic sensing and vibration data. |
| Multiple failures | Combining any of the failure states into a multi-failure state. For example, 2 loose bolts at minimum loading. |

Table 4 – Induced failure States

Each failure state will be recorded for 1 minute and labelled, then sliced into 1s sample blocks. For each block, signals will be extracted resulting in 60 unique sets of data the model cam be trained on.

In a live sensing application, ML outputs will be structured as a percentage of total health, for example, a healthy but old engine may exhibit a probability split of 80% normal, 10% loose mounting and 10% off axis reading.

## Non Machine Learning Strategy

Inducing bearing failure for machine learning labels is unrealistic. Therefore, bearing faults will be monitored by gating frequency energy bands deviating from a normal operating level established at the beginning of sensing and referenced against known fault signatures [3].

## System Output

Simple threshold gating will determine the output of the system. Logic that combines the ML and non-ML strategies will trigger warning messages associated with decreasing motor health. For example, a “Healthy” message is displayed when both the ML and non-ML strategies have not surpassed any thresholds. Alternatively, “Engine Health: 80%” can be tracked using a scoring system.